Urban Air Quality Index Modeler

Abstract

Air pollution is a major environmental concern, especially in urban areas where PM2.5 levels significantly affect public health. Accurate prediction of PM2.5 concentration is essential for effective environmental monitoring and urban planning.

This project develops a machine learning-based model to predict PM2.5 levels using pollutant indicators (PM10, SO₂, NO₂, CO, O₃), meteorological factors (Temperature, Pressure, Dew Point, Rainfall, Wind Speed), and engineered temporal features such as Month and Hour of Day.

Three regression models—Linear Regression, Random Forest, and XGBoost—were implemented and evaluated using RMSE, MAE, and R² score. XGBoost showed the best performance and was deployed using a Streamlit web application for real-time prediction. The project supports data-driven decision-making aligned with SDG 11: Sustainable Cities and Communities.

Introduction

Rapid urbanization and increasing vehicular emissions have led to rising air pollution levels. PM2.5 is one of the most harmful pollutants due to its ability to penetrate deep into the respiratory system. While monitoring systems measure pollution levels, predictive systems are necessary for proactive management.

This project applies machine learning techniques to forecast PM2.5 concentration using historical air quality and meteorological data. Temporal features such as Month and Hour of Day are included to capture seasonal variations and peak traffic patterns. The final model is deployed as a web application to enable easy and real-time prediction.

Problem Statement

Urban air pollution levels fluctuate significantly due to the complex interplay between traffic density, fluctuating weather patterns, and seasonal transitions. While modern monitoring systems are effective at providing real-time data, they often fall short in offering the predictive foresight necessary for proactive public health management. To bridge this gap, there is a critical need for a reliable machine learning-based system capable of accurately forecasting PM2.5 concentrations by capturing the intricate, non-linear relationships between various pollutants and atmospheric factors. By integrating these sophisticated modeling techniques into an accessible web platform such a system would empower stakeholders and the general public with actionable, real-time predictions to better navigate environmental risks

​

Methodology

Data Preprocessing and Analysis: Conduct thorough cleaning and exploratory data analysis (EDA) on air quality datasets to identify key trends and ensure data integrity.

Model Development and Comparison: Implement and compare three distinct machine learning algorithms—Linear Regression, Random Forest, and XGBoost—to determine the most effective approach for predicting PM2.5 levels.

Model Performance Evaluation: Assess the accuracy and reliability of each model using standardized metrics, including Root Mean Square Error (RMSE), Mean Absolute Error (MAE), and R² score.

Interactive Model Deployment: Develop and launch a real-time prediction interface using the Streamlit framework, allowing users to interact with the best-performing model.

Support for Sustainable Urban Planning: Align the project outcomes with UN Sustainable Development Goal 11 to provide actionable insights for building safer, more resilient, and sustainable cities

System Architecture / Workflow

Process Flow:

A step-by-step breakdown from raw data input to prediction output.

Exploratory Data Analysis (EDA)

Correlation Analysis:

How strongly traffic volume relates to PM2.5.

​Visualizations:

Inclusion of Heatmaps, Scatter plots, and Time-series graphs.

Model Development & Results

Algorithm Selection:

Why you chose specific Regression models (e.g., Random Forest or Linear Regression).

​Performance Metrics:

Discussion of R^2 scores and Mean Absolute Error (MAE).

Comparison:

If multiple models were tested, show which performed best.

SDG 11 Impact & Analysis

Relevance:

How this model helps city planners reduce the environmental impact of cities (Target 11.6).

​Sustainability:

Discussion on how data-driven decisions lead to "Smarter Cities."

Project Management (President's Section)

​Team Roles:

Distribution of the 16 members into Data, Model, and Documentation squads.

​GitHub Workflow:

Summary of branching (data-eng, model-dev) and commit frequency.

​

Conclusion & Future Scope

Summary:

Final thoughts on model reliability.

Future Work:

Scaling the model for real-time sensor integration or mobile app alerts.

References & Appendix

Data Sources: Links to the datasets used.

Meeting Proof: Screenshots of Daily Syncs and the Attendance Sheet.